Network Engineering Capstone
Functionality Report

# Introduction

Provide a functionality report detailing the 10 test-case scenarios used to verify the utility of your network project. Seven of the test-case scenarios must be from the provided predefined list, with the remaining three test cases created by you. The functionality report should be written so that a networking peer could replicate the steps for a successful test of your networking solution.

| Student Name | Emily Parker |
| --- | --- |
| WGU Student ID | 011200197 |
| WGU Student Email | epar255@wgu.edu |

# Test Case #1: Device Discovery and Reachability

Your network solution must include multiple network segments with access controls that allow traffic from a device on one network to access the resources of a device on another network. Similarly, there must be devices on one network that cannot access resources on a different network.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

Config subnets ONLY. Point them to test case #2 for ACL configuration. Don’t configure DHCP, just configure static IP’s on the routers/L3 switches. DHCP is for Test Case #4.

static route from r1 to r4

The functionality of this test case is exhibited by pinging from  R1, at Fenrir site 1 to R4, at Fenrir site 2. For traffic to successfully flow from R1 to R4, there needs to be proper routes configured. Static routes were configured and were verified to be working correctly. Tools used include the “IP route” command, and the ping command which utilizes ICMP echo requests/replies. Allowing certain devices to access/not access resources will be configured in test case #2 (ACL’s) instead of here. Relevant devices include R1, R2, R3, and R4. Relevant subnets include 192.168.7.0 /24, 192.168.10.0 /24, and 192.168.11.0 /24.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method used here was the ping command. By pinging R4 from R1, I was able to verify that traffic is able to be forwarded between site 1 and 2. Metrics of success include the success rate of the pings being 5/5, meaning there was no loss, and all packets reached the destination.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

After static IP’s were assigned, the next step to test reachability was to have routes configured. Test case #8 will cover OSPF configuration for dynamic routing. For now static routes will do, they will be deleted afterwards.

Ping from 192.168.7.0 to 192.168.11.0 should still fail, routes haven’t been configured on the other routers.

It does fail, this is expected behavior.

After configuring the other two static routes on R2 and R3, the ping between Fenrir 1 and Fenrir 2 was successful.

# Test Case #2: Administering an Access Control List for Guest Access

Your network must utilize an access control list that allows guest access. Guest access should be limited to internet traffic only.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

PC’S ON THE GUEST SUBNETS HAVE STATIC IP’S ONLY TEMPORARILY FOR THIS TEST CASE. THEY WILL ACQUIRE DYNAMIC IP’S LATER IN TEST CASE #4.

Functionality exhibited here includes the PC’s on each Guest subnet being able to access the Internet and only the Internet. The router labeled “Internet”; hostname “ISP-Router” is really a stand-in. Packet Tracer has limited functionality, so the PC’s cannot actually access the Internet. Access to internal network resources is denied by the use of a standard numbered ACL. Commands I have used here include the “ip access-list [number]” command to create the ACL, “access-list [number] permit/deny [IP] [wildcard mask]” to add an ACE to the list, and the “ip access-group [number] in/out” command to apply the ACL inbound or outbound on the interface. It is very important to use a wildcard mask instead of a standard subnet mask, or else it will not work correctly. For clarity, you must create the ACL in global config mode, and apply it in interface config mode.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method I used to verify functionality of this test case is to use the ping command to verify the PC’s on the guest subnet cannot access internal network resources. I applied the ACL outbound on ISP-Router’s F1/0 interface so that when PC10 and PC11 try to communicate to the internal network, they are simply blocked from doing so. On top of that, ONLY those two subnets are blocked when ISP-Router checks its outbound ACL for matches. Pings from the guest PC’s to the internal network were successfully blocked, as shown below in the Process List.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

Commands to add ACE’s to the ACL, then a show command to list all current IP access-lists. NOTE: The command to create a standard numbered ACL is “ip access-list [number]”. These commands must be run in global config mode.

Switching to interface F1/0, and applying the ACL outbound. The command here is a bit different, as you specify “access-group” rather than “access-list” This is done in interface config mode.

Verifying that pings from PC10 to ISP-Router’s F1/0 interface fail. This behavior is expected, and wanted. This proves that the configuration and application of the ACL is correct.

Again verifying that pings were unsuccessful, this time from PC11 to ISP-Router’s F1/0 interface. This behavior is expected, and wanted.

# Test Case #3: Security Compliance—Log-in Banners

Display a log-in banner when accessing each device on the network. The log-in banner should notify users of an acceptable use policy (AUP) or other security-based policies when attempting to log into the network.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The functionality exhibited in this test case is meant to show the company Acceptable Use Policy to users when logging onto network devices. Relevant tools include the “banner motd” command, followed by a delimiter character of your choosing. This will allow you to write your login message, and then end it with the delimiter character. Relevant devices include every L2 switch, L3 switch, and router in the network. If Packet Tracer supported the functionality, the login banner would also be present on every end user device in the network.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method in this case is very simple. Open a CLI connection on the selected device, and the AUP will show up.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

The first step in creating login banners for the network devices is to use the “banner motd” command, followed by a delimiter of your choosing. This will allow the device to know which part is the command and which part is the login banner. NOTE: “motd” stands for message of the day.Then you can write your banner, and add the delimiter again.

SW1 login banner configuration

SW2 login banner configuration

SW3 login banner configuration

L3SW1 login banner configuration

R1 login banner configuration

R2 login banner configuration

R3 login banner configuration

R4 login banner configuration

R5 login banner configuration

R6 login banner configuration

L3SW3 login banner configuration

SW4 login banner configuration

SW5 login banner configuration

SW6 login banner configuration

As seen here. when logging onto a device, the login banner that I have configured is displayed. Packet Tracer does not have the functionality required for configuring login banners on the PC’s.

# Test Case #4: Accessing External Resources—Routing and Traffic Security

User devices on your network should have dynamic addresses that are assigned through DHCP unless they provide a service that requires a static address. You must also have at least one network resource that requires a static address.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The desired functionality here is to have all end user devices to have IP addresses assigned dynamically by the use of DHCP. This way, devices that no longer need an IP address(ex. Device is powered off, or no longer in use) can release their IP’s back to the DHCP server for use by another device. This creates a smooth and dynamic IP allocation and release process. If necessary, devices can request a reservation so that they receive the same IP upon each boot up, for example. Network devices should receive static IP’s so we can reliably access and perform configuration changes with a previously known IP. For example, if each time you SSH into a device and the IP is different, it creates unneccsary confusion and effort. We want the process to be smooth and simple. Our guest networks should not communicate with the internal network, so they will receive IP’s from ISP-Router, configured as a DHCP server. The hosts on the internal network will receive IP’s from our internal DHCP servers. Relevant subnets include 192.168.3.0 /24, 192.168.4.0 /24, 192.168.5.0 /24, 192.168.18.0 /24, 192.168.19.0 /24 PC0, PC1, PC2, PC3, PC4, PC5, PC6, PC7, PC8, PC9, PC10, and PC11.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method I used to verify functionality of DHCP was to use the commands “ipconfig”, “ipconfig /release”, and “ipconfig /renew.” The ipconfig command by itself will show information such as your current IP address, subnet mask, and default gateway. The release addition to the command allows the client to relase their IP back to the DHCP server. Finally, the renew part of the command allows the client to query the DHCP server for a new IP address. First I used ipconfig by itself to verify that the client does in fact not have a current IP address, or has an APIPA address, which would point to the DHCP server not working. I then added /release to be positive the client did not have an address. Then, I used the /renew command to allow the client to acquire an IP address.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

Configuring the DHCP Server1 pool to start at 192.168.3.7, and configuring the default gateway given to clients, 192.168.3.5.

Configuring DHCPServer2

Configuring DHCPServer3

Configuring DHCPServer4

Configuring DHCPServer5

Configuring DHCPServer6

Configuring DHCPServer7

Configuring DHCPServer8

PC0 has successfully acquired an IP address, subnet mask, and default gateway from the DHCP server. The “ipconfig /release” command releases the IP back to the DHCP pool so that another host can use it. The “ipconfig /renew” command starts the DHCP DORA process to acquire an IP address.

PC1 acquiring an address from the DHCP server

PC2 acquiring an address from the DHCP server

PC3 acquiring an address from the DHCP server

PC4 acquiring an address from the DHCP server

PC5 acquiring an address from the DHCP server

PC6 acquiring an address from the DHCP server

PC7 acquiring an address from the DHCP server

PC8 acquiring an address from the DHCP server

PC9 acquiring an address from the DHCP server

PC10 acquiring an address from the DHCP server

PC11 acquiring an address from the DHCP server

# Test Case #5: Layer 2 Link Redundancy and Spanning Tree Protocol (802.1w)

Enable and manage the Spanning Tree Protocol to establish redundant Layer 2 paths while avoiding possible loops and broadcast storms. Identify the Layer 2 devices that will become the root bridge. <<<<<<<< main part of this test case

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The functionality of this test case is to prevent the network from experiencing layer 2 loops. This is accomplished by the use of STP: Spanning Tree Protocol. The enhanced version, Rapid STP(RSTP) is the more common version. Actually, Cisco switches use a version of STP called Per-VLAN Spanning Tree(PVST), which is being used in this demonstration. STP accomplishes a loop-free layer 2 network by blocking certain switch ports, preventing broadcast storms from occurring. By default, switches automatically run STP, so there is no need to apply any configurations. Relevant devices include SW1, SW2, SW3, SW4, SW5, SW6, and L3SW1 and L3SW2’s switchports. Relevant tools include the “show spanning-tree” and “show version” commands.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method I used to verify the root bridge selection process was to use the “show version” and “show spanning-tree” commands. Show-spanning tree will allow you to see which switch is the root bridge. If a switch is the root bridge, it will say directly “this bridge is the root bridge”. To verify that the election process was correct, I used the show version command to see each switch’s base MAC address. I then verified that the root bridge’s MAC was indeed the lowest, which means that the root bridge election process was carried out correctly.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

SW1’s MAC address. The “base ethernet MAC address” is the actual device’s MAC address, not the address of any single port on the switch.

SW3’s MAC address

L3SW1’s MAC address

SW2’s MAC address. As we can see by the results of “show version” and “show spanning-tree”, SW2 has the lowest MAC address, therefore it wins the root bridge election.

SW4’s MAC address

As shown by “show version” and “show spanning-tree”, this switch has the lowest mac address, and it is the root bridge for Fenrir Site 2.

SW5’s MAC address

SW6’s MAC address

L3SW3’S MAC address

# Test Case #6: Edge Device Syslog and NTP

Configure perimeter devices to generate system logs that capture unwanted traffic. Additionally, those perimeter devices should utilize Network Time Protocol (NTP) for clock synchronization.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The functionality of this test case pertains to first configuring NTP, so that time can be synchronized across devices. This is important for then configuring Syslog, which needs the correct time in order to have accurate logs. Accurate logs are helpful, for example, in an investigation in which you are trying to correlate events and the sequence in which they occurred.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

I shutdown an interface on R3 to verify that logging to the external server is working. This is not a great idea for a real network, but for our purposes, it shows that the configuration is correct, and SyslogServer1 is working as intended.

Here I use the show ntp associations command to verify that R3 has associated with its NTP server 192.168.8.2, L3SW1.

Verifying that R4 has associated with its NTP server 192.168.13.1, L3SW3.

I did the same test for SyslogServer2 as I did for SyslogServer1. Logs are being sent to the external server. The server is functioning as intended.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

It is important to configure NTP before Syslog so that time across the network will be consistent in the logs. First, I configure the timezone on L3SW1, which will be our NTP server. With the command “clock timezone [timezone] and the offset -5, meaning the time will be 5 hours behind UTC, the default timezone on Cisco devices. We want our NTP server to have accurate time, that is crucial for NTP to work correctly. If the server time is wrong, then all client devices using that server for time synchronization will be wrong too. Using the “clock set” command will allow you to set the device’s software clock time.

Configuring L3SW1 to become the NTP server for Fenrir Site 1. The “ntp master” command configures the device to act as an NTP server with the default stratum being 8. Other devices can now sync to this server with the command “ntp server [ip]”.

Configuring Fenrir Site 1’s edge router, R3, to use L3SW1 as its NTP server. The IP 192.168.8.2 is L3SW1’s F0/3 interface. The command “show ntp associations” will do just that, it will show which NTP servers that R3 has associated with. If necessary, several NTP servers could be configured as peers to take the place of L3SW1 if it went down.

Configuring clock and setting timezone on L3SW3.

Configuring L3SW3 to act as Fenrir Site 2’s NTP server.

Configuring R4 to use L3SW3 as its NTP server. End of NTP configuration.

Start of syslog configuration. The command “logging [ip]” will configure the device to send its syslogs to an external server, the IP specified in the command. I configure R3 to send them to Fenrir Site 1’s syslog server, SyslogServer1.

Configuring R4 to send its sylogs to an external server 192.168.21.2, which is Fenrir Site 2’s syslog server, SyslogServer2.

# Test Case #7: Basic Network Segmentation at Layer 2 via VLANs and 802.1q

Your network traffic should be segmented per department or service function at Layer 2 to enhance security and reduce network congestion at the switching layer while allowing segmented traffic to traverse between switches (VLAN trunking).

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The functionality of this test case involves configuring VLAN’s on the L2 switches(access and trunk ports), configuring VLAN’s and SVI’s on the L3 switches, and verifying proper functionality. Relevant tools include the L2 and L3 switches, VLAN 110, 120, 130, subnets 192.168.3.0 /24, 192.168.4.0 /24, 192.168.5.0 /24, and their respective end hosts to verify connectivity.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method I used below in the Process List was a combination of the ping and the tracert commands. Ping was used to simply test connectivity between devices. Tracert was used to test the functionality of VLAN’s, and because of inter-VLAN routing, tracert was able to verify that the first hop was the originating device’s default gateway, and not the destination device.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

Creating and assigning VLAN 110 to interfaces f0/4, f0/5, and f0/6 on SW1. Instead of one at a time, I use the “range” command to do multiple. The “switchport mode access” command makes the interfaces access ports, not trunk ports. “Switchport access vlan [number] assigns the interfaces to the VLAN. However, if the specified VLAN does not already exist on the switch, this command will also create the specified VLAN for you.

Here I am doing the same configuration on SW2 for interfaces f0/5, f0/6, and f0/7. This configuration is for VLAN 120.

SW3 configuration for VLAN 130

SW4 configuration for VLAN 110

SW5 configuration for VLAN 120

SW6 configuration for VLAN 130

Configuration of trunk ports

Configuring a trunk port on SW1 f0/1. “Switchport mode trunk” works the same as “switchport mode access” only now for a trunk port. I also configure the allowed VLAN’s, including 110, 120, and 130.

Configuring which VLAN’s are allowed on the trunk. NOTE: In the screenshots there is no “add” in the command, ex. “switchport trunk allowed vlan add 110”. I have realized and fixed this in the actual configuration. Below there will be the output of “show interfaces trunk” on each switch to verify that VLAN’s 110, 120, and 130 are all allowed.

SW2 trunk port configuration on interface f0/1

SW2 trunk port configuration on interface f0/2

SW3 trunk port configuration on interface f0/1

SW4 trunk port configuration on interface f0/1

SW5 trunk port configuration on interface f0/1

SW2 trunk port configuration on interface f0/2

SW6 trunk port configuration on interface f0/1

L3SW1 SVI configuration for VLAN 110. To create an SVI, I used the interface vlan[number] command, followed by giving the SVI an IP address, and turning the interface on by using the “no shut” command.

L3SW1 SVI configuration for VLAN 120

L3SW1 SVI configuration for VLAN 130

Acc and trunk ports successfully configured.

Configure site 2 SVI’s HERE:

Configuring an SVI for VLAN 110 on L3SW3

Configuring an SVI for VLAN 120 on L3SW3

Configuring an SVI for VLAN 130 on L3SW3

From the earlier note:

SW1 “show interfaces trunk”

SW2 “show interfaces trunk”

SW3 “show interfaces trunk”

SW4 “show interfaces trunk”

SW5 “show interfaces trunk”

SW6 “show interfaces trunk”

The thing to note here is the tracert command that I ran with the destination of the DHCP server at 192.168.4.1. We can see that the first hop is from PC0 to it’s default gateway, 192.168.3.254, and then to the DHCP server at 192.168.4.1. If the DHCP server were in the same broadcast domain, that first hop to the default gateway would not be needed, since they would be in the same subnet. Since I have separated the LAN into different VLAN’s, inter-VLAN routing is needed, which is why traffic first went to PC0’s default gateway. This shows that the VLAN’s are operational and working as intended for Fenrir Site 1.

The tracert from PC6 to DHCP Server2 at 192.168.4.6 goes through PC6’s default gateway first, just like from PC0 to DHCP Server1, proving that the VLAN/SVI configuration is functioning properly.

# Test Case #8: Basic or Advanced Networking - OSPF

## Custom Test Case

Define a custom test case to be run within your network project aligned to your specific organizational need or opportunity identified in Task 1. The custom test case should be equivalent in scope and requirements to the predefined test cases and pertain to basic or advanced networking.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The purpose of this test case is to configure OSPF on the routers and L3 switches upstream interfaces for both Fenrir Site 1 and 2, and to verify that routes are being shared, routers are becoming OSPF neighbors, and adjacencies are being formed. Relevant devices include R1, R2, R3, R4, R5, R6, L3SW1, and L3SW2. Relevant subnets include 192.168.6.0 /24, 192.168.7.0 /24, 192.168.8.0 /24, 192.168.10.0 /24, 192.168.12.0 /24, 192.168.13.0 /24, 192.168.14.0 /24, and 192.168.15.0 /24.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method I used to verify functionality were several commands. These include the “show ip ospf neighbor” command, and the “show ip route” command. The command “show ip ospf neighbor” will show the devices OSPF neighbors, current dead timers, and also the interface. For more granularity, you can use the “show ip ospf interface [interface]” command. This will show you information such as network type,  neighbor count, whether the device is designated, etc. I used the “show ip route” command to verify that OSPF routes were actually being shared between neighbors. These commands are fairly simple but allow for easy verification that OSPF, and dynamic routing in general, is functional.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

Configuring OSPF first on L3SW1. From global configuration mode, you can use the “router ospf [process ID]” command to enter OSPF configuration mode. The process ID is entirely local, unlike EIGRP, in which they would have to match in order for routers to become neighbors. OSPF routers however, can have different process ID’s and still become neighbors. The network command’s syntax is as such: “network [matching interface IP that you want to activate OSPF on] [wildcard mask] area [area]. Since I’m configuring single area OSPF, it’s easiest to use area 0. This command specifies which interface you want to activate OSPF on, not which network. The wildcard mask is essentially an inverse of a subnet mask, meaning a /24, like which I used here, would be 0.0.0.255 instead of 255.255.255.0. Once I configure OSPF on a neighboring router, you can expect to see a Syslog message indicating they have become neighbors, as long as the configuration is correct and matching(hello and dead timers, unique router ID’s, etc).

R1 OSPF configuration. R1 has not become neighbors with R2, as I have not configured OSPF on R2 yet. R1 and L3SW1 however, are neighbors.

R2 OSPF configuration. Now, R2 and R1 have become neighbors.

R3 OSPF configuration. R3 is neighbors with R2, but not yet with R4, as R4 has not been configured yet.

R4 OSPF configuration

R5 OSPF configuration. R5 is neighbors with R4, but not yet with R6, as R6 has not been configured yet.

R6 OSPF configuration

L3SW3 OSPF configuration

# Test Case #9: IPSec

## Custom Test Case

Define a custom test case to be run within your network project aligned to your specific organizational need or opportunity identified in Task 1. The custom test case should be equivalent in scope and requirements to the predefined test cases and pertain to IPSec.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

After attempting to configure an IPsec VPN between R3 and R4, I realized that the routers do not have the capability to acquire the security license necessary for the configuration. Therefore, instead of potentially creating unwanted problems in the network, I will create another small network to showcase the IPsec VPN configuration.

Configuring the security license on Router0.

Configuring the security license on Router1.

Configuring an ACL on R3 for the tunnel between R3 and R4.

Configuring an ACL on R4. This is for the tunnel R4 will have established with R3

# Test Case #10: Network Security

## Custom Test Case

Define a custom test case to be run within your network project aligned to your specific organizational need or opportunity identified in Task 1. The custom test case should be equivalent in scope and requirements to the predefined test cases and pertain to network security.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Network Troubleshooting

Discuss how you analyzed the network to identify, troubleshoot, and resolve issues during development or when ensuring functionality of the test cases.